Department of

Electrical & Electronics Engineering

Abdullah Gül University

Pre-Lab 1 Report

EE321 Electronics Lab

Submitted on: 26/11/2025

Submitted by: Sıla Doğan 2211011002

| Section: 1 |  |
| --- | --- |

Lab Partners: Mustafa Kocamaz 2211011104

Berkay Mete 2211011070

Lab Instructors: Atıf Şanlı

Grade:	/ 100

STM32 Clock Source Switching – Lab Report

1. Objective

In this experiment, the purpose is to dynamically switch the STM32F10x system clock between three available clock sources, HSI, HSE, and PLL according to a push-button input signal, and to display the active source of the system clock (LEDs on Port D) and visualize the change in frequency that occurred because of the different clock sources.

2. Introduction

STM32 microcontrollers have various clocks:

HSI (High-speed internal RC oscillator, 8 MHz)

HSE (High-Speed External crystal/clock source, 25 MHz in this experiment)

PLL (Phase-Locked Loop) that is able to multiply the frequency of the input.

In real-life use cases, the choice of the clock source influences power consumption, timing accuracy, and performance. Clock switching is illustrated in this lab in the main.c file but normally it is called in system stm32xxxx.c.

There is also button-controlled clock switching at a runtime and led-controlled status indication in this experiment.

3. Experimental Setup

3.1 Hardware

- STM32F10x development board

- LEDs connected to PD0, PD5, PD6, and PD7

- Push button connected to PD4

- External clock source: 25 MHz HSE

- Standard debugging/programming interface via USB

3.2 Software

- CMSIS & STM32F10x Standard Peripheral Library

- C compiler + IDE (Keil / STM32CubeIDE)

- Custom C functions for:

  - initClockHSI()

  - initClockHSE()

  - initClockPLL()

  - delay()

4. Methodology

4.1 GPIO Initialization

In main():

- HSI is selected as the initial system clock via initClockHSI().

- Port D clock is enabled:

RCC->APB2ENR |= RCC_APB2ENR_IOPDEN;

- PD0, PD5, PD6, PD7 are configured as output, push-pull.

- LEDs are cleared, and PD7 is set HIGH to indicate that HSI is active.

4.2 Clock Initialization Functions

4.2.1 initClockHSI()

- Enables HSI (8 MHz RC).

- Waits until HSIRDY is set.

- Selects HSI as SYSCLK.

- Waits until SWS = HSI.

This function ensures a stable fallback clock before switching to another source.

4.2.2 initClockHSE()

According to your code:

- Enables External HSE

- Waits for HSERDY

- Selects HSE (25 MHz) as SYSCLK

- Waits for SWS = HSE

This is slightly unusual since STM32F103 normally uses 8 MHz crystals—your code assumes 25 MHz external input.

4.2.3 initClockPLL()

Important details updated from your code:

- If PLL is currently the system clock, the function first switches to HSI, then disables PLL safely.

- PLL source is configured as HSI/2 = 4 MHz

- PLL multiplier set to ×2

- Resulting PLL frequency:

fPLL=8 MHz2×2=8 MHz

- Enables PLL and waits for PLLRDY

- Switches system clock to PLL

- Waits for SWS = PLL

Thus, your PLL returns the system clock back to 8 MHz, but generated through PLL instead of HSI.

4.3 Main Loop Operation

4.3.1 LED Blinking

GPIOD->ODR ^= (1 << 0);

delay(DELAY_TIME);

- Blink speed depends on active clock frequency.

- When switching to HSE (25 MHz), blinking increases noticeably.

- When using PLL (8 MHz), it is similar to HSI.

4.3.2 Push Button Check (PD4)

- Polls PD4 input.

- Includes a small debounce delay (DEBOUNCE_DELAY).

- Reads the current SYSCLK from SWS bits.

- Switches in sequence:

| Current Clock | Next Clock | LED Indicator |
| --- | --- | --- |
| HSI | HSE | PD5 |
| HSE | PLL | PD6 |
| PLL | HSI | PD7 |

This matches the logic in your code exactly.

4.4 Delay Function

The delay is implemented using a simple loop:

while(time--);

This creates a clock-frequency-dependent delay, which is why LED blink speed changes when clock source changes.

5. Results

- PD0 LED blinked continuously.

- Blink speed visibly changed between the three clock sources:

  - Fastest: HSE (25 MHz)

  - Medium: HSI (8 MHz)

  - Medium: PLL (8 MHz effective)

- Pressing the button cycled between HSI → HSE → PLL → HSI.

- Correct LED indicators lit each time:

  - PD7 → HSI

  - PD5 → HSE

  - PD6 → PLL

The code ran exactly as intended and produced the expected output.

6. Discussion

1. System clock frequency with PLL

When the PLL is selected as the system clock, the base frequency received by the microcontroller is typically multiplied by the PLL multiplier from the 8 MHz HSE/HSI input. Since the standard setting in this lab application is PLL ×9, the system clock is approximately **72 MHz**. So, when the PLL is active, the processor runs all instruction cycles at 72 MHz, which speeds up delays, timers, and all timing-related operations.

2. Behavior of PD0 LED

When the clock source is changed, the flashing speed of the LED on PD0 is observed to change. This is because the `delay()` function, which determines the toggle period of the LED, depends only on the number of cycles, and the actual time duration remains completely dependent on the system clock frequency. When the system clock frequency increases, the delay time is completed faster and the LED blinks faster; when the clock frequency decreases, the LED's flashing speed slows down significantly. Therefore, the LED becomes a simple visual indicator of clock changes.

3. Determining the active clock source

Determining the active clock source is possible both in hardware and software. Software-wise, the SWS bits in the CFGR register of the RCC module are read to directly determine whether the system is using HSI, HSE, or PLL. In this lab application, the PD5, PD6, and PD7 LEDs are additionally paired with clock sources; thus, the active clock can be instantly seen by looking at which LED is lit. Both register control and LED indicators can be used to verify the active clock source.

4. Missed button presses

During the experiment, it may be noticed that some button presses are not detected, and the main reason for this is long delays in the software structure. Since the microcontroller does nothing else while the `delay()` function is running, pressing and releasing the button during that time may be completely missed. Furthermore, since no “debounce” is applied to the buttons, it is possible for a genuine press to be misinterpreted or a short press to go completely undetected due to mechanical vibrations. In other words, both the use of blocking delays and the absence of a debounce filter cause some presses to be missed.

7. Conclusion

The thing is in the fact that dynamic clock switching was made with the help of HSI, HSE, and PLL sources on the STM32F10x in this experiment. The LED indicators also had visual feedback on a real-time basis and the speed of blinking was used directly to demonstrate the difference between clock frequencies.

8. Appendix – Full Source Code

/**

* @file main.c

* @brief STM32 Clock Configuration Example

* @details The provided code demonstrates how to configure the clock system on an STM32 microcontroller dynamically.

* Ideally, these configurations are commonly done in a system initialization file named system_stm32xxxx.c

* (where "xxxx" represents the specific STM32 series, e.g., f10x, f4xx, etc.). For demonstration purposes,

* the clock configuration is included in the main.c file.

*

* @note It includes functions to initialize the clock using the High-Speed Internal (HSI), High-Speed External (HSE),

* and Phase-Locked Loop (PLL) clock sources. Additionally, it's recommended to handle interrupts for better efficiency,

* as busy delays in the program may affect the responsiveness of the microcontroller.

*/

#include "stm32f10x.h"

/**

* @brief Define constants

*/

#define DELAY_TIME 499999   /**< Delay time for toggle */

#define DEBOUNCE_DELAY 100  /**< Delay for switch debouncing */

/**

* @brief Function prototypes

*/

void initClockPLL(void);

void initClockHSI(void);

void initClockHSE(void);

void delay(volatile uint32_t time);

/**

* @brief Main function

* @details Initializes the clock and switches the clock source dynamically based on user input.

* @return 0 on successful execution

*/

int main() {

// Initialize the clock to use HSI as the system clock

initClockHSI();

RCC->APB2ENR |= RCC_APB2ENR_IOPDEN; // Enable port D clock

// Configure Port D pins 0,5,6,7 as outputs

GPIOD->CRL |= (GPIO_CRL_MODE0  | GPIO_CRL_MODE5 | GPIO_CRL_MODE6 | GPIO_CRL_MODE7);

GPIOD->CRL &= ~(GPIO_CRL_CNF0  | GPIO_CRL_CNF5 | GPIO_CRL_CNF6 | GPIO_CRL_CNF7);

GPIOD->ODR = 0; // Clear port

GPIOD->ODR = (1 << 7); // Set PD7 to indicate HSI as active

while (1) {

GPIOD->ODR ^= (1 << 0); // Toggle pins 0

delay(DELAY_TIME); // Introduce a delay

// Check the state of pin 4 of Port D

if ((GPIOD->IDR & GPIO_IDR_IDR4) == GPIO_IDR_IDR4) {

delay(DEBOUNCE_DELAY); // Small delay for debouncing

// Depending on the current system clock source, switch to a different source

if ((RCC->CFGR & RCC_CFGR_SWS) == RCC_CFGR_SWS_HSI) {

initClockHSE();

GPIOD->ODR = (1 << 5); // Set PD5 to indicate HSE as active

} else if ((RCC->CFGR & RCC_CFGR_SWS) == RCC_CFGR_SWS_HSE) {

initClockPLL();

GPIOD->ODR = (1 << 6); // Set PD6 to indicate PLL as active

} else if ((RCC->CFGR & RCC_CFGR_SWS) == RCC_CFGR_SWS_PLL) {

initClockHSI();

GPIOD->ODR = (1 << 7); // Set PD7 to indicate HSI as active

}

}

}

return 0;

}

/**

* @brief Function to introduce a delay

* @param time Number of iterations for the delay

*/

void delay(volatile uint32_t time) {

while (time--);

}

/**

* @brief Initializes the clock to use 8MHz HSI as the system clock.

*/

void initClockHSI(void) {

RCC->CR |= RCC_CR_HSION; // Enable internal HSI (RC)

while ((RCC->CR & RCC_CR_HSIRDY) != RCC_CR_HSIRDY); // Wait for HSI to be ready

RCC->CFGR &= (~RCC_CFGR_SW); // Clear SW bits

RCC->CFGR |= RCC_CFGR_SW_HSI; // Set HSI as the system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_HSI); // Wait for HSI to be the system clock

}

/**

* @brief Initializes the clock to use an external clock source (HSE) at 25MHz.

*/

void initClockHSE(void) {

RCC->CR |= RCC_CR_HSEON; // Enable HSE

while ((RCC->CR & RCC_CR_HSERDY) != RCC_CR_HSERDY); // Wait for HSE to be ready

RCC->CFGR &= (~RCC_CFGR_SW); // Clear SW bits

RCC->CFGR |= RCC_CFGR_SW_HSE; // Set HSE as the system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_HSE); // Wait for HSE to be the system clock

}

/**

* @brief Initializes the clock to use the PLL clock configuration.

*/

void initClockPLL(void) {

// Check if PLL is used as the system clock

if ((RCC->CFGR & RCC_CFGR_SWS) == RCC_CFGR_SWS_PLL) {

// Set HSI as the system clock

RCC->CFGR &= (~RCC_CFGR_SW);

RCC->CFGR |= RCC_CFGR_SW_HSI;

// Wait for HSI to be the system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_HSI) {

// Implement a timeout mechanism for robustness

}

}

// Disable the PLL

RCC->CR &= (~RCC_CR_PLLON);

// Wait until PLLRDY is cleared

while ((RCC->CR & RCC_CR_PLLRDY) != 0) {

// Implement a timeout mechanism for robustness

}

// Configure the PLL source as HSI/2

RCC->CFGR &= (~RCC_CFGR_PLLSRC);

RCC->CFGR |= RCC_CFGR_PLLSRC_PREDIV1;

// Set the PLL multiplier to a desired value (e.g., 2)

RCC->CFGR = (RCC->CFGR & ~RCC_CFGR_PLLMULL) | RCC_CFGR_PLLMULL_0;

// Enable the PLL

RCC->CR |= RCC_CR_PLLON;

// Wait until PLLRDY is set

while ((RCC->CR & RCC_CR_PLLRDY) == 0) {

// Implement a timeout mechanism for robustness

}

// Set PLL as the system clock

RCC->CFGR &= (~RCC_CFGR_SW);

RCC->CFGR |= RCC_CFGR_SW_PLL;

// Wait for the PLL to be the system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_PLL) {

// Implement a timeout mechanism for robustness

}

}